Electrical operating mechanism series

>> DAM1(ABB) Electrical operating mechanism

The electrical operating mechanism is available remote close of MCCB

The types of electrical operating mechanism are shown in table 2

>> Types of electrical Operating Mechanism:   Table 2

| Rated current of frame size Inm | Types of electrical Operating Mechanism |  |  |  |
| --- | --- | --- | --- | --- |
|  | CD1 | CD2 | CD3 | CD |
| 63A | - | CD2-63 | - | - |
| 100A | - | CD2-100 | - | - |
| 160A | - | CD2-160 | - | - |
| 250A | CD1-250 | CD2-250 | - | CD-250 |
| 400A | CD1-400 | - | - | CD-400 |
| 630A | - | - | CD3-630 | CD-630 |
| 800A | - | - | CD3-800 | CD-800 |
| 1250A | - | - | CD3-1250 | CD-1250 |
| 1600A | - | - | CD3-1600 | CD-1600 |

>> Description of selection:

Customer can make selection of the electrical operating mechanism according to the price and types e.q.If you want to order CD type 630A electrical operating mechanism ,having rated operational voltage AC 220V with total quantities of 12 ,you should write in this way in your order :CD-630A/AC220V,12sets.

>> CD1 type electrical operating mechanism

CD1 type electrical operating mechanism

Direct-acting and manual close/open of MCCB

Equipment with emergency opening button

Customer can fit padlock on it for MCCB in open position (up to a max of 3 padlock with max 6mm dia).It can be supplied witch.

One key and one lock for one MCCB

One key and two lock for three MCCB

Two key and three lock for three MCCB

Suitable for MCCB

Inm=250A

Inm=400A

The specific type should be stated clearly in your order

Selectable accessories

>> Characteristic

Characteristic of CD1 type electrical operating mechanism          Table 3

| Electrical accessories |  |  |  | Electrical operating mechanism |  |
| --- | --- | --- | --- | --- | --- |
| Range of supply voltage |  |  |  | (0.85-1.1)X Us |  |
| Rated control supply voltage | Power supply | AC 50Hz |  | 220V | 380V |
|  |  | Power loss | Irrush power consumption | 510VA | 510VA |
|  |  |  | Normal load | 360VA | 360VA |
|  |  | DC |  | 110V | 220V |
|  |  | Power loss | Irrush power consumption | 510W | 510W |
|  |  |  | load | 360W | 360W |
| Closing time |  |  |  | 0.1S |  |
| Opening time |  |  |  | 0.1S |  |

CD2 type electrical operating mechanism

>> Direct-acting and manual close/open of MCCB

Equipment with emergency opening button Suitable for MCCB

Inm=63A Inm=100A Inm=160A Inm=250A

The specific type should be stated clearly in your order

Selectable accessories

CD2 type electrical operating mechanism